Computer Architecture Lab

(EL-2012)

LABORATORY MANUAL

FALL 2026

(Lab#06)

Use of Venus Simulator for RISC-V Procedure Execution

Student Name: __________ Roll No: ________________ Section: _A__

Date performed: _____________, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

____________________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad | Version: 2.1 |
| --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah | Updated: Spring 2026 |

Learning Objectives: - To use Venus simulator for simulation of RISC-V assembly language program - To monitor the register contents after execution of RISC-V assembly instructions - To execute procedures and nested functions using Venus simulator. ## Tool Used:

Venus Simulator ## Assembly Language: Refer to earlier Labs #06 “Venus Simulator for Register Level Execution of Arithmetic and Memory Instructions” and #07 “Venus Simulator for Register Level Execution of Control Instructions” for background information on Venus simulator and its usage, specially: - RISC-V Registers - Assembly Instructions - Data Declarations - Load / Store Instructions - Indirect and Based Addressing ## Procedures: - Procedures help decompose applications into smaller modules - Assembly language procedures require - Storage for local variables - Arguments must be passed in and return values passed out - Execution must continue after the call ## Procedure Steps - Place parameters where procedure can access them - Transfer control to the procedure - Save any needed context of calling program - Obtain storage needed for the procedure - Execute the desired task - Make results available to the calling program - Restore any needed context of calling program - Return control to the point of origin ## Supporting Procedures RISC-V registers: - Registers are the fastest place to hold data in a computer - Dedicated RISC-V registers to support procedure calls - a0 – a7 (Argument registers to pass parameters. At most eight function arguments can be passed) - a0 – a1 (Value registers to return values. At most two values can be returned) - ra (Return address register to return execution to the point of origin) RISC-V instructions: - jal (jump-and-link) - saves the address of the following instruction in register ra (why not current instruction?) - Jumps to an address to service the procedure call - The link stored in register ra is called the return address (what is it?) - jalr (jump-and-link register) - jumps to the address stored in the register ra (why not a label as in jump?) - Used by called procedure to return control to calling program - Procedure Call - The calling program (caller) puts the parameter values in a0 - a7 and uses jal X to jump to procedure X (callee). Note the correct syntax in the RISC-V reference card. - The callee then - performs the calculations - places the results in a0 - a1, and - returns control to the caller using jalr ra. Note the correct syntax in the RISC-V reference card. Stack: - Stack is a LIFO data structure. A natural way of temporarily store data for procedures as well as call/return information - Stacks historically grow down in memory - Stack operations: push, pop - RISC-V stack - The sp register should always point to the top of the stack - Stack grows from higher addr to lower address - RISC-V does not provide push and pop instructions. They must be implemented by the programmer. Stack Push operation: - To push data onto the stack - make room for the data by moving sp down - allocate storage. - place data into the allocated storage. - example: push registers s0, t0 and t1 onto the stack. RISC-V Code: addi sp, sp, -4 # decrement stack pointer to allocate stack space sw s0, 0(sp) # saved (preserved) registers need to be saved by callee # t0/t1 are temp (non-preserved) registers and need not be saved. # If caller needs their contents after a procedure call, they should # be saved by caller itself. Stack Pop operation: - To pop data from the stack - any stored data may be accessed by its position relative to sp - example: retrieve contents of registers s0, t0 and t1 from stack RISC-V Code: lw s0, 0(sp) # saved (preserved) registers need to be saved # t0/t1 are temp (non-preserved) registers and need not be restored addi sp, sp, +4 # increment stack pointer to free up stack - As you can see it is possible to access any data that has been placed on the stack through the sp register. Remember that preserving registers on stack is a shared responsibility between caller and callee, as shown in the figure below:

Tasks:

Complete following tasks in Lab.

- Examine why the below procedure is not displaying correct value in t1 register. Fix it.

.text

main:

li a0, 1

li s0, 2500

jal ra, procedure			# call procedure

add t1, s0, a0			# result should be 1003+2500

li a0,1						# setup system call for display (command in a0)

mv a1, t1					# data to display in a1

ecall						# displays result on console

procedure:

li s0, 3

li t0, 1000

add a0, s0, t0

jalr zero, ra, 0			# return

- Run given example on Venus Simulator and explain the code.

addi s0, zero, 5

addi t1, zero, 10

addi t0, zero,15

addi sp, sp, -12

sw   s0, 0(sp)

sw   t0, 4(sp)

sw   t1, 8(sp)

lw   t4, 8(sp)

lw   t2, 4(sp)

lw   t3, 0(sp)

addi sp, sp, 12

- Write RISC-V assembly code for given C code:

int main(){

x=EqSolver(a,b);

}

int EqSolver (int a, int b){

return (a+b)(a-b);

}

- Write RISC-V assembly code to find the area and perimeter of square. You need to write two procedures (i.e. AreaFinder and PerimeterFinder ) and call both functions in main module.

- Write RISC-V assembly code for given C code:

int main(){

y=EqSolver(x);

}

int EqSolver (int x){

return (x^3+5x^2+6x);

}

- Implement the following C code in RISC-V assembly using Venus.

int main(int n, int k){

if (k == 1)

y=F1(n,k);

else if (n > k)

y= F2(n);

else

y= F3(n);

}

int F1 (int x,int z){

return (x2 + z2);

}

int F2 (int z){

return (z3);

}

int F3 (int w){

return (w+5);

}

- Implement the following C code in RISC-V assembly using Venus.

int main(int n, int k){

if (k == 0)

y=fun1(n);

else if (n == k)

y= fun2(n);

else

y= fun3(n);

}

int fun1 (int x){

return (x2);

}

int fun2 (int z){

return (z3);

}

int fun3 (int w){

return (w+1);

}

- Implement the following C code in RISC-V assembly using Venus.

Without Recursion:

int main(){

y=fib(n);

}

int fib(int n)

{

int a=0,b=1,c=i;

if(n==0)

return a;

else 	{

for(i=2;i<=n;i++)

{

c=a+b;

a=b;

b=c;

}

return b;

}}

With Recursion:

int main(){

y=fib(n);

}

int fib(int n)

{

if (n==0)

return 0;

else if (n == 1)

return 1;

else

return fib(n−1) + fib(n−2);

}