Computer Architecture Lab

(EL-2012)

LABORATORY MANUAL

Fall 2026

(Lab#03)

Apply Behavioral-level Verilog to Design and Simulate Combinational Modules

Student Name: _______________________________ Roll No: ________________________ Section: ______

Date performed: _________________________, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

____________________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 5.1 |  |  |
| --- | --- | --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |  |  |

Learning Objectives: - Verilog Behavioral Modeling of Combinational Circuits. Tool Used:

ModelSim Starter Edition 10.5b Behavioral Level Modeling Verilog provides designers with the ability to describe the design functionality in an algorithmic manner. In other words, the designer describes the behavior of the circuit. The abstraction in this modeling is as simple as writing the logic in C language. Verilog behavioral models contain procedural statements that control the simulation and manipulate variables of the data types previously described. The activity starts at the control constructs initial and always. Each initial statement and each always statement starts a separate activity flow. All activity flows are concurrent, allowing the user to model the inherent concurrence of hardware. Synthesis converts Verilog HDL models of hardware down to gate-level implementations automatically and maps these into the target technology. Control Constructs or Structured Procedures: - Initial Block: //sample code integer cnt; initial begin cnt = cnt + 2; #20 cnt = cnt + 5; #20 cnt = cnt + 2; end - Initial block starts at simulation time zero and all statements are executed once. - Execution of statement starts from begin and stops when the end of the block is reached. - Initial block cannot be synthesized. - Always Block: //sample code reg [0:7] cnt; always @ * begin cnt <= cnt + 2; end - always block starts at simulation time 0 and repeats forever with changes in elements in sensitivity list (indicated by @ symbol). - Execution of statement starts from begin and stops only when wait statement in encountered. - It is a synthesizable construct of Verilog. Assignments in Verilog HDL: There are two types of assignments i.e. continuous assignment and procedural assignment in Verilog. The continuous assignment is used to drive net data type variables using the 'assign' statements whereas procedural assignments are used to drive reg data type variables using initial and always block statements. Continuous assignments are non-synthesizable. Procedural assignments: Procedural assignments can be used only within the structured procedures always and initial blocks. The left-hand side of assignment should be one of the following: - Register. - Bit-select of register. - Part-select of reg, integer, or time data type. - Memory word. - Concatenation of any of the above. Procedural assignments statements are further classified into two types: blocking (Example 1) and non-blocking (Example 2) procedural assignments. - Blocking procedural assignments: //Example 1 begin a = 1; #10 a = 0; #5 a = 4; end During the simulation, this block will be executed in 15 time units. At time 0 the 'a' variable will be 1, at time 10 the 'a' variable will be 0, and at time 15 (#10 + #5) the 'a' variable will be 4. - Non-blocking procedural assignments: //Example 2 begin a <= 1; #10 a <= 0; #5 a <= 4; end During the simulation this block will be executed in 10 time units. At time 0 the 'a' variable will be 1, at time 5 the 'a' variable will be 4, and at time 10 the 'a' variable will be 0. ## Differences: ## Continuous Assignment; ## Procedural Assignment. - Continuous assignment can only drive wire or tri data type. Which means left-side data type should be net data type. - Continuous assignments should appear outside procedures (always, initial, functions, tasks, etc). - It executes each time the right hand side expression changes. It overrides any procedural assignment. - A wire should be assigned only once using continuous assignment. - Only combinational logic can be modeled using Continuous Assignment.; - Procedural Assignment can drive only reg., integer, real and time data type. Which means left-side data type cannot be nets. - Procedural Assignment should appear inside procedures. - It can model both combinational and sequential logic.. Conditional (if-else) statement: - The condition (if-else) statement is used to decide whether a statement is executed or not. The keywords if and else are used to make conditional statement. - Syntax: if (condition_1) statement 1; else if (condition_2) begin statement 2; end else statement 3; - Case statement: The case statement is a multi-way decision statement that tests whether an expression matches one of the expressions and branches accordingly. Keywords case and endcase are used to make a case statement. The case statement syntax is as follows. Syntax: case (expression) case 1: statement_1; case 2: statement_2; case 3: statement_3; default: default_statement; endcase - Loop statement: There are four types of looping statements in Verilog:  for loop  while  forever  Repeat Syntax:

Procedure: Step 1: Open ModelSim; launch from start tab or desktop icon. Step 2: Create a new project. You may use a standard naming convention, e.g., “Lab_06”. Select your project location/directory. Advisable to create subdirectory for ease of reference. Avoid use of ‘- ‘in file or module names. Step 3: If you have a source file to add into the project, then click on Add Files. Else click on Create File. Name your file e.g., “Task_01” or “Top”, etc. Make sure you select “Verilog” as your Simulator language. Step 4: Now open editor and define the input/output ports of your module (e.g., a, b are inputs, o is output). If you have groups of bits to work with, you may bind them together as a bus. May leave these blank and declare later in module definition too. Click OK. For simulation: Writing the Testbench: Testbench is used to check the functionality of a program. Write the testbench below the main module. Simulating the Design: To simulate your design, click on Simulation -> Run Simulation and Generate waveform.

Source code Example: - Always Block: module basic (a, b); input [7:0] a; output reg [7:0] b; always @ * begin b <= a + 2; end endmodule - Initial Block: module basic (a, b, c); input [7:0] a; output reg [7:0] b,c; always @ * begin b <= a + 2; end initial begin c = a + 2; // #20 c = a + 5 // #20 c = a + 2; end endmodule - Blocking assignments: module basic (a,b); // module name and variables to be used in program input [3:0] a; //define input output reg [3:0] b; //define outputs initial begin b = 0; #20 b = 5; //Blocking assignments #10 b = 10; end endmodule //end of program - Non- Blocking assignments: module basic (a,b); // module name and variables to be used in program input [3:0] a; //define input output reg [3:0] b; //define outputs initial begin b <= 0; #20 b <= 5; // Non-Blocking assignments #10 b <= 10; end endmodule //end of program

Lab Tasks:

- Write Verilog code for a 4-bit adder/subtractor. The circuit should perform addition or subtraction depending on the control signal. Simulate the results using a testbench.

- Write Verilog code for 2x1 MUX having below mentioned operations. Simulate your program through testbench.

Note: Select, A and B are 1-bit inputs. Y is one bit output.

| Select | Output (Y) |
| --- | --- |
| 0 | A & B |
| 1 | A ¦ B |

- Write Verilog code for a 2-bit comparator that compares two 2-bit binary numbers and outputs the result as equality, greater than, or less than. Simulate the circuit with a testbench.