Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#_03__)

Write Lab Title Here

Student Name: ___Abdulmoeiz_______ Roll No: _______24i-6510_________ Section: __CE A_

Date performed: ____1 Sep____, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

This lab task was about writing behavioural level Verilog code for adder and subtractor . Control flow statements were used to perform addition/subtraction. Later to test, its test bench coce was written and simulation was run to check the behaviour of wave form which matches the expected result

Code:

module addsub (a, b, sub, result); input [3:0] a, b; input sub; output reg [4:0] result; always @(*) begin if (sub == 0) result <= a + b; else result <= a - b; end endmodule module addsub_testbench; reg [3:0] a, b; reg sub; wire [4:0] result; addsub uut (.a(a), .b(b), .sub(sub), .result(result)); initial begin a = 4'd5; b = 4'd3; sub = 0; #20; a = 4'd5; b = 4'd3; sub = 1; #20; end endmodule

Output:

Task 2:

Description:

In this task , behavioural level Verilog code for 2x1 multiplexer was written. In order to make2x1 multiplexer use control flow statements. Later to test the Verilog code, its test bench was written and simulation of code was run and check the behaviour of wave form which matches the expected result

Code:

module MUX2x1(a,b,c,s); input a,b,s; output reg c; always @* begin if(s==0) c=a&b; else c=a¦b; end endmodule module MUX2x1_testbench; reg a,b,s; wire c; MUX2x1 uut( .a(a),.b(b),.s(s),.c(c) ); initial begin a=0;b=1;s=0; #50; a=0;b=1;s=1; #50; a=1;b=1;s=0; #50; a=1;b=1;s=1; #50; a=0;b=0;s=0; #50; a=0;b=0;s=1; #50; end endmodule

Output:

Task 3:

Description:

Code:

Output:

// Import output here 👇