Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#_02__)

Write Lab Title Here

Student Name: ___Abdulmoeiz_______ Roll No: _______24i-6510_________ Section: __CE A_

Date performed: ____24  August____, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

In this task I designed a Full adder circuit. For this task I built a Half Adder module first and then made another full adder module where 2 Half Adders were made to implement sum and carry operation. Later on, to simulate the circuit a test bench module was made with different combinations of inputs. This task was implemented through both gatelevel and dataflow level.

Code:

module ha(sum, carry, a, b); input a, b; output sum, carry; xor(sum, a, b); and(carry, a, b); endmodule module FA(SUM, Cout, A, B, Cin); input A, B, Cin; output SUM, Cout; wire Sum1, C1, C2; ha ha1(Sum1, C1, A, B); ha ha2(SUM, C2, Sum1, Cin); or(Cout, C1, C2); endmodule module FA_testbench; reg A,B,Cin; wire SUM, Cout; FA FA1(SUM,Cout,A,B,Cin); initial begin A = 0; B = 0; Cin = 0; #10 A = 0; B = 0; Cin = 1; #10 A = 0; B = 1; Cin = 0; #10 A = 0; B = 1; Cin = 1; #10 A = 1; B = 0; Cin = 0; #10 A = 1; B = 0; Cin = 1; #10 A = 1; B = 1; Cin = 0; #10 A = 1; B = 1; Cin = 1; end endmodule DATAFLOW LEVEL MODELLING: module ha(sum, carry, a, b); input a, b; output sum, carry; assign sum = a^b; assign carry = a&b; endmodule module FA(SUM, Cout, A, B, Cin); input A, B, Cin; output SUM, Cout; wire Sum1, C1, C2; ha ha1(Sum1, C1, A, B); ha ha2(SUM, C2, Sum1, Cin); or(Cout, C1, C2); endmodule module FA_testbench; reg A,B,Cin; wire SUM, Cout; FA FA1(SUM,Cout,A,B,Cin); initial begin A = 0; B = 0; Cin = 0; #10 A = 0; B = 0; Cin = 1; #10 A = 0; B = 1; Cin = 0; #10 A = 0; B = 1; Cin = 1; #10 A = 1; B = 0; Cin = 0; #10 A = 1; B = 0; Cin = 1; #10 A = 1; B = 1; Cin = 0; #10 A = 1; B = 1; Cin = 1; end endmodule LOGISIM CIRCUIT:

Output:

TESTBENCH WAVEFORMS: LOGISIM Circuit TruthTable :

Task 2:

Description:

This task was about designed a 3x8 decoder. This task was required to use multiple input and output variables. Also, we included the “enable” bit as a function as well. The code was implemented in gatelevel and dataflow level and a testbench module was written to test our 3x8 decoder and generate it’s waveforms. The circuit was also deisgned on Logisim to ensure correct working via the TruthTable analysis.

Code:

GATELEVEL MODELLING: module decoder( a, b,c,en,o0,o1,o2,o3,o4,o5,o6,o7); input en,a,b,c; output o0,o1,o2,o3,o4,o5,o6,o7; wire na,nb, nc; not(na,a); not(nb,b); not(nc,c); and(o0, en, na, nb, nc); and(o1, en, na, nb, c); and(o2, en, na, b, nc); and(o3, en, na, b, c); and(o4, en, a, nb, nc); and(o5, en, a, nb, c); and(o6, en, a, b, nc); and(o7, en, a, b, c); endmodule module Decode_TB; reg en,a,b,c; wire o0,o1,o2,o3,o4,o5,o6,o7; decoder d1(a,b,c,en,o0,o1,o2,o3,o4,o5,o6,o7); initial begin en = 0; a = 0; b = 0; c = 0; #10 en = 1; a = 0; b = 0; c = 0; #10 en = 1; a = 0; b = 0; c = 1; #10 en = 1; a = 0; b = 1; c = 0; #10 en = 1; a = 0; b = 1; c = 1; #10 en = 1; a = 1; b = 0; c = 0; #10 en = 1; a = 1; b = 0; c = 1; #10 en = 1; a = 1; b = 1; c = 0; #10 en = 1; a = 1; b = 1; c = 1; end endmodule DATAFLOW LEVEL MODELLING: module decoder( a, b,c,en,o0,o1,o2,o3,o4,o5,o6,o7); input en,a,b,c; output o0,o1,o2,o3,o4,o5,o6,o7; wire na,nb, nc; assign na =~a; assign nb =~b; assign nc =~c; not(nc,c); assign o0 = na& nb& nc &en; assign o1 = na& nb& c &en; assign o2 = na& b& nc &en; assign o3 = na& b& c &en; assign o4 = a& nb& nc &en; assign o5 = a& nb& c &en; assign o6 = a& b& nc &en; assign o7 = a& b& c &en; endmodule module Decode_TB; reg en,a,b,c; wire o0,o1,o2,o3,o4,o5,o6,o7; decoder d1(a,b,c,en,o0,o1,o2,o3,o4,o5,o6,o7); initial begin en = 0; a = 0; b = 0; c = 0; #10 en = 1; a = 0; b = 0; c = 0; #10 en = 1; a = 0; b = 0; c = 1; #10 en = 1; a = 0; b = 1; c = 0; #10 en = 1; a = 0; b = 1; c = 1; #10 en = 1; a = 1; b = 0; c = 0; #10 en = 1; a = 1; b = 0; c = 1; #10 en = 1; a = 1; b = 1; c = 0; #10 en = 1; a = 1; b = 1; c = 1; end endmodule LOGISIM CIRCUIT DESIGN:

Output:

TESTBENCH WAVEFORM: LOGISIM CIRCUIT TRUTHTABLE:

Task 3:

Description:

// Write description of the task here 👇 (max. five to seven lines)

Code:

Output:

// Import output here 👇

Task 2:

Description:

// Write description of the task here 👇 (max. five to seven lines)

Code:

Output:

// Import output here 👇

Task 2:

Description:

// Write description of the task here 👇 (max. five to seven lines)

Code:

Output:

// Import output here 👇